该型号产品描述：（包括：产品的综合描述、产品功能、产品应用、产品参数等）

Gate station

HZM-801CY

Features:

1. Luxury design, Maximum capacity is unlimited

2. Touch Response keyboard, Color Video

3. LCD follow-up operation menu

4. Digital Audio Encoding

5. Can call/talk with Guard Center and the Guard Center can remotely unlock

6.Unlock with password and cards (IC card: Type A, I; ID card: Type A, E),

disarm by cards

7. Language select

8. Burglar alarm(optional)

9. CCD with night infrared compensation

10. TCP/IP Agreement, network, remote system upgrade

Specifications:

- Display: LCD

- Working temperature:-30 - 55

- Working humidity: 45% - 95%

- Working current: ≤400mA

- Quiescent current: ≤250mA

- Input Voltage: +DC15V

- Protection grade: IP33

- Conversation time: About 120s

- Camera:1/3”SONY SHAD CCD

- Resolution: 420TV Line

- IC/ID card reading distance: ≤30mm

- Dimension:117x335x80mm

- Bottom case dimension: 92x335x65mm

- Network transmission protocol: TCP/IP

- Cable requirement:1*CAT5 8-core cable,1*RVV3 X 1.0mm as power cable.

- Network Port: RJ45

- Transmission range: router or PBX less than 100m, FOT more than 100m